Slide 1.

There are two part to my project. The first part is building the Beowulf cluster. The second part is using the cluster to simulate quantum confinement.

Slide 2. Introduction

So, having read the project title you probably have two questions. What is a Beowulf cluster? And What is quantum confinement?

So, a Beowulf cluster is a computing cluster where each node is connected on a local area network and each node on the network has libraries and programs installed on it which allow processing power to be shared across all the nodes. A Beowulf cluster is usually built using consumer-grade electronics. But your final result should be a high-performance parallel computing cluster.

Second question, what is quantum confinement? So, quantum confinement is an effect that occurs relating to the electronic and optical properties of a material when you look at a small enough sample. Such as less than 10nm of material.

Slide 3. Beowulf Cluster

Having built the cluster, I ran a benchmark called LINPACK. This is the same benchmark that has been used since 1993 by top500.org who document the 500 fastest supercomputers in the world at the time. They update the list every 6 months as that is needed to keep up with the rate at which computers are getting faster.

So, is this cluster a super computer? For todays standards, no. the fastest super computer in Ireland called the Kay supercomputer, as you can see from this graph is way faster than the cluster. But I took the 50th super computer from 1997 which was the year I was born, and this cluster is twice as fast as it. My phone is the one Plus 5t which has an 8-core processor that runs at 2.6GHz is 100 times slower than the cluster. A recentish dual core machine got a score of 70. This machine in particular is one from a lab here in the college.

But if this cluster had been built in the year 2001, which was the year the twin towers collapsed the it would have been 360th on the top500 list.

So, to set up the cluster some of the basic things required are password less ssh, a shared directory for the nodes to share files and a protocol to enable the communication between the nodes. Which in this case I will be using a mixture of openmpi and a built-in functionality of the language I am using which is Julia.

Slide 4. Julia

So according to the Julia website these are the aims of the language. They want it to be fast, dynamic optionally typed, general, easy to use and open source. But the three aspects of the language that I noticed were. If it’s supposed to be fast well then it makes sense to want to use it for a high-performance operation. Second, I found it very easy to use. This is the atom IDE developed by GitHub but with a special package installed called uber-juno which converts Atom into a Julia programming environment. It has all the usual things like a file structure, code tabs, your workspace which also shows your plots and finally the console output section. It also has GitHub integration which is handy to use to transfer the code from my laptop to the cluster using the online repository.

Finally, the open source aspect, last week I was creating a 3D graph and they z-axis label wasn’t working and I later found this was because it wasn’t actually implemented yet. But the package was updated last week and now I have z-axis labels for the specific type of plotting function I was using.

More info on the Julia language can be found at their website.

Slide 5. Wave packet construction

A wave in quantum mechanics is the probability of finding a particle in that region of space. Waves have real and imaginary parts. And as you can see from this graph this wave suggests that there is a particle located at around 0.4 units.

This is the Julia code for this equation. As you can see it uses similar notation to MatLab for elementwise operations but the one thing that I found noticeably different was that you have to include the dot after the exponential function for the code to execute.

Slide 6. 1D Leapfrog

So, the previous slide did not account for time, it was just a stationary wave. So, when you start to include time in the equation you have to start using different methods. One of these methods is the leapfrog method. It works by first having selected a timestep. Then it uses this timestep to calculate the real and imaginary parts of the wave equation and adding them together to form the wave that you see here. It is called the leapfrog method as first it will calculate the real part at time t delta_t, next the imaginary part will be calculated at delta_t-delta_t/2 and delat_t+delta_t/2. Or a half step back and a half step forward. So, the real part is calculated at full steps and the imaginary part is calculated at half steps. So, to get the full equation the real and imaginary parts leapfrog over each other to form the full equation.

In this graph you can see what happens when the wave hits a potential barrier which in this case is a cliff.

Slide 7. 2D Leapfrog

A similar method is used here to be able to visualise what happens to the wave when it hits a cliff or a wall in two dimensions.

Slide 8. Conclusion

So, in conclusion what left to do for this project? First, I would like to make the cluster smaller such as taking out the motherboards and power supplies and mounting them on some sort of shelf or rack system.

Next the code needs to be parallelised so that it can operate better on the cluster as at them moment it is only being run on one machine.

And finally make the cluster more powerful by adding additional nodes but this may not be required as it is already pretty fast.